## Research positioning and novelty boundary

GPU acceleration of the Method of Moments is not, by itself, novel. Peng and Nie demonstrated GPU acceleration of MoM calculations in 2008; Lezar and Davidson subsequently treated GPU-accelerated MoM and GPU LU factorization; Topa et al. mapped RWG-based MoM to CUDA; and Shi and Jin later accelerated integral-equation operations including matrix filling and matrix-vector products on GPUs [7]–[11]. [1]

Likewise, periodic MoM is well established. Cwik and Mittra applied MoM to periodic conducting/resistive patch arrays; Singh et al. addressed the slow convergence of periodic Green-function series; Michalski and Mosig developed layered-media Green-function machinery for integral equations; and Dardenne and Craeye formulated MoM for infinitely periodic structures containing metallic and dielectric objects [2]–[5]. More recently, Dessy et al. presented a hybrid spectral/spatial periodic-MoM strategy specifically for efficient metasurface analysis in 2025 [6]. [2]

Nor should the paper claim that Python, GPU acceleration, metasurface analysis, or inverse-design compatibility are individually unprecedented. Bempp-cl provides a Python boundary-element environment with OpenCL execution on CPUs and GPUs, including Maxwell-scattering examples [12]. TORCWA is a particularly important neighboring work: it is a Python/PyTorch, GPU-accelerated rigorous coupled-wave analysis implementation that supports gradient-based metasurface optimization [13]. [3]

The defensible novelty is therefore the combination and specialization:

A periodic MoM solver deliberately engineered as a high-throughput, Python-native GPU engine for metasurface unit-cell S-parameter generation, with Floquet-periodic electromagnetic modeling, GPU-parallel matrix construction and solution, batch/HPC execution, and an interface intended for direct use in data-driven and physics-guided inverse-design pipelines.

The literature surveyed here did not identify a peer-reviewed work that unambiguously combines all of those elements in one periodic-MoM unit-cell workflow. That should be expressed as the outcome of a literature survey, not as an absolute “first-ever” claim. The existence of recent periodic-MoM work [6], Python/GPU boundary-element software [12], and GPU-differentiable RCWA [13] makes such qualification particularly important. [4]

Commercial EM packages are proprietary and their deployment on HPC/GPU infrastructure is governed by vendor licensing and entitlement models, but actual cost depends on institutional agreements, products, and hardware configuration. The scientifically safer claim is that a self-controlled solver removes proprietary-solver licensing from the marginal cost and scheduling path of each generated sample, while commercial full-wave software can remain the validation reference. Official commercial-solver documentation confirms that GPU/HPC execution exists within vendor-specific licensing frameworks. [5]

One further scope point is essential. The theoretical draft below assumes, unless your implementation already does more, electrically thin conducting patterns embedded in or printed on plane-stratified media. That is a natural target for surface-integral-equation MoM. Arbitrary three-dimensional dielectric meta-atoms require equivalent electric/magnetic surface-current formulations such as PMCHWT, a volume-integral equation, or another extension; those capabilities must not be implied unless they actually exist. Dardenne and Craeye's work illustrates that periodic formulations combining metal and dielectric bodies require additional machinery beyond the simplest PEC EFIE [5]. [6]

## Abstract and introduction

Abstract— Data-driven metasurface design increasingly requires large numbers of accurate electromagnetic simulations for surrogate-model training, generative inverse design, optimization, and candidate validation. Repeated generation of such datasets using general-purpose commercial full-wave solvers can introduce practical limitations associated with automation, computational throughput, cluster deployment, and proprietary licensing. This paper presents a Python-native, GPU-compatible Method of Moments framework for high-throughput electromagnetic analysis of periodically repeated metasurface unit cells. The formulation combines a Floquet-periodic surface integral equation, unit-cell current discretization, periodic layered-medium Green functions, GPU-parallel impedance-matrix construction, GPU-based complex linear algebra, and modal post-processing for extraction of reflection and transmission S-parameters. The resulting solver is designed to operate both as a standalone forward simulator and as a physics engine embedded within machine-learning and inverse-design workflows. Validation is performed against a commercial full-wave solver using canonical and parametrically varied metasurface geometries, together with mesh-convergence. The implementation is further benchmarked against a CPU reference to quantify matrix-assembly time, solution time, end-to-end acceleration, GPU-memory scaling, and dataset-generation throughput. For [Ntest] validation structures, the proposed solver achieves [complex S-parameter error], an end-to-end acceleration of [speed-up], and a sustained throughput of [unit cells/hour]. Rather than introducing GPU acceleration of MoM as an isolated concept, the work targets an integrated computational-electromagnetics workflow for scalable metasurface dataset generation and physics-guided inverse design.

Index Terms— computational electromagnetics, method of moments, metasurface, periodic structures, GPU computing, CUDA, S-parameters, Floquet modes, inverse design, deep learning, high-performance computing.

Introduction— Metasurfaces achieve prescribed electromagnetic transformations through spatially engineered subwavelength inclusions, making the electromagnetic response of an individual periodically repeated unit cell a fundamental quantity in many design workflows. For a specified geometry, material stack, frequency, incidence angle, and polarization, the design problem commonly reduces at the unit-cell level to determining quantities such as complex reflection and transmission coefficients. When the structure is treated as infinitely periodic, the fields and induced currents satisfy Bloch/Floquet conditions, allowing a single cell to represent the infinite array. Periodic integral-equation and MoM formulations for conducting arrays have a long history, including Cwik and Mittra's treatment of periodic conducting and resistive patches and later developments for more general periodic metallic/dielectric structures [2], [5]. [7]

For thin metallic or impedance-sheet structures, an integral-equation formulation is particularly attractive because the unknown current is confined to the electromagnetic scatterer's surface rather than throughout the surrounding free-space volume. Arbitrarily shaped conducting surfaces can be discretized using Rao-Wilton-Glisson basis functions [1], while plane-stratified substrates can be represented through the appropriate layered-medium Green functions [4]. These characteristics make MoM a natural candidate for printed frequency-selective surfaces and many microwave metasurface unit cells, although it is not universally preferable to finite-element, finite-difference, or Fourier-modal methods for every metasurface class. [8]

The computational difficulty arises when this forward problem must be solved not once but tens of thousands—or potentially many more times—during design-space exploration or data generation. A conventional surface MoM discretization produces a dense impedance matrix. With N current unknowns, explicit matrix storage scales nominally as O(N2), while a conventional dense direct factorization scales nominally as O(N3). In addition, evaluating the pairwise Green-function interactions required for matrix construction can dominate computation for moderate-sized repeated solves. Previous research therefore explored GPUs for both MoM matrix evaluation and dense linear algebra [7]–[11]. [1]

The same computational burden has acquired renewed importance with the growth of machine-learning-assisted electromagnetic design. Neural-network-based forward surrogates and inverse models have been demonstrated for nanophotonic scattering and inverse design [14], [16], while generative models have been used to produce metasurface geometries from desired spectra [15]. These methods shift a substantial fraction of computational expense to the data-generation stage: the neural network is inexpensive to evaluate after training, but its training pairs must first be obtained from electromagnetic calculations. [9]

This dependence is not merely hypothetical. Ghorbani et al. used full-wave simulations of randomly generated metasurface unit cells to construct a learning dataset for wideband automatic metasurface design; their workflow coupled data-generation scripts to CST Microwave Studio to obtain the required electromagnetic responses [17]. Such approaches are effective, but they demonstrate how a data-driven workflow can inherit the simulation throughput, automation, and licensing constraints of the external full-wave engine used to label its samples. [10]

General-purpose commercial packages remain valuable because of their broad material models, meshing capabilities, multiphysics support, mature user interfaces, and extensive validation. The objective here is therefore not to replace commercial electromagnetic software for all applications. Instead, the motivation is to develop a narrower solver whose numerical formulation is tailored to a repetitive workload: periodic metasurface unit-cell analysis followed by extraction of S11 and S21. By controlling the complete numerical stack, large parameter sweeps can be submitted directly to locally available or institutional GPU infrastructure, while selected cases remain cross-validated using established commercial software.

Several related directions already exist and define the novelty boundary of the present work. Periodic MoM itself is established [2]–[6]; GPU-based MoM has been reported extensively [7]–[11]; Bempp-cl demonstrates that Python and accelerated boundary-element computing can coexist [12]; and TORCWA provides a GPU-accelerated Python Fourier-modal solver together with gradient-based metasurface optimization [13]. Consequently, neither “MoM on a GPU” nor “a Python metasurface solver” constitutes a sufficient research contribution by itself. [11]

Contribution statement suitable for the final paragraph of the Introduction.
The contribution of this work is an integrated periodic-MoM framework designed specifically for high-throughput metasurface unit-cell characterization. In contrast to earlier GPU-MoM studies primarily concerned with accelerating general scattering calculations, the present implementation couples Floquet-periodic electromagnetic modeling, direct unit-cell S-parameter extraction, GPU-parallel impedance assembly and linear algebra, and batch-oriented Python interfaces intended for large-scale dataset generation. The same forward interface can be called directly from machine-learning and inverse-design workflows, permitting the MoM model to function both as an offline data generator and as an online physics evaluator. The solver is validated against [CST/HFSS] over [N] unit-cell configurations and benchmarked on CPU and GPU hardware to quantify accuracy, computational scaling, and dataset-generation throughput.

That contribution statement is defensible provided the paper supplies the validation and performance evidence named in it.

## Theoretical formulation

Draft manuscript text—Theoretical Formulation.
Consider a two-dimensional periodic metasurface defined by primitive lattice vectors a1 and a2, with lattice translations

Rmn=ma1+na2,  m,n∈Z.

The derivation below adopts the ejωt convention. A plane wave incident on the structure has tangential wave vector ktinc and electric field

Einc(r)=E0pe-jkinc⋅r,

where p denotes the polarization vector. Periodicity reduces the infinite-array problem to one unit cell because any electromagnetic quantity F satisfying the Floquet condition obeys

F(ρ+Rmn,z)=F(ρ,z)e-jktinc⋅Rmn.

Periodic integral-equation formulations of this type have long been used for arrays of conducting or resistive patches and for more general periodic structures [2], [5]. [7]

For an electrically thin perfectly conducting element occupying the surface Sc inside the reference cell, the tangential electric-field boundary condition is

n×Einc(r)+Escat(r)=0,  r∈Sc.

The scattered field is related to the induced surface current Js through the periodic electric-field integral operator,

Escat=LperJs.

For the adopted ejωt convention, a representative mixed-potential form is

Lper{Js}=-jωμSc​GAper(r,r')Js(r') dS'+1jωϵ∇Sc​Gϕper(r,r')∇s'​⋅Js(r') dS',

with the scalar expression generalized to dyadic and layered-medium Green functions as required by the substrate. For printed structures on multilayer substrates, the appropriate layered Green function is essential; replacing it by a free-space kernel would omit substrate-mediated interactions. Michalski and Mosig provide a foundational treatment of multilayer Green functions in integral-equation formulations [4]. [12]

The periodic Green function may be written conceptually as a phase-weighted lattice sum,

Gper(r,r')=m=-∞∞n=-∞∞G​r,r'+Rmne-jktinc⋅Rmn.

A direct evaluation of such lattice sums can converge slowly, which has motivated convergence-acceleration, Ewald, spectral-domain, and mixed spatial/spectral approaches. Singh et al. specifically treated acceleration of series representing the free-space periodic Green function [3], while recent periodic-MoM work by Dessy et al. combines spectral- and spatial-domain ideas for efficient metasurface analysis [6]. [13]

The surface current is expanded in a finite set of basis functions,

Js(r)=n=1NInfn(r),

where In are the unknown expansion coefficients. For an unstructured triangular mesh, the RWG basis is a natural choice because it enforces normal-current continuity across adjacent triangular facets and supports arbitrary conducting geometries [1]. If your actual implementation instead uses rooftops, pulse bases, or a pixel-grid formulation, (6) and the associated discussion should be changed rather than claiming RWG. [14]

An important implementation detail for periodic RWG discretization is that edges intersecting opposite boundaries of the unit cell cannot simply be treated as open edges. Their basis-function contributions must be associated across periodic boundaries with the appropriate phase factor from (1). This is where the Floquet condition enters not only the Green function but also the discretized current continuity for basis functions crossing the cell boundary, consistent with the periodic integral-equation framework [2], [5]. [7]

Using Galerkin testing, substitution of (6) into (2) yields

n=1NZmnIn=Vm,

where

Zmn=fm,Lper{fn},

and

Vm=-fm,Einc.

The complete discretized problem is therefore

ZI=V.

The accuracy of Z depends strongly on the evaluation of self, near-neighbor, and regular interactions. The GPU implementation must therefore reproduce whatever singularity extraction or high-order quadrature is used by the CPU reference; simply evaluating all interactions using the same ordinary quadrature rule would not constitute a robust MoM implementation.

For a resistive or impedance-sheet metasurface, the PEC condition can be generalized to

Et=ZsJs,

which introduces the corresponding surface-impedance contribution into the MoM matrix. For volumetric dielectric inclusions, however, (2)–(10) are not sufficient. Equivalent electric and magnetic surface currents or a volume-integral formulation must be introduced. Periodic metallic/dielectric structures have been treated with more general MoM formulations [5]; such capability should only be claimed if it has actually been implemented. [6]

### Floquet modes and extraction of scattering parameters

Let b1,b2 denote reciprocal-lattice vectors satisfying

ai⋅bj=2πδij.

The tangential wave vector of Floquet harmonic (p,q) is

kt,pq=ktinc+pb1+qb2,

and, in medium l,

kz,pq(l)=kl2-kt,pq2,

where the radiation branch is selected such that propagating waves radiate away from the structure and evanescent harmonics decay away from it.

Once the current coefficients are known, the spectral content of the induced current can be evaluated through

Jpq=Sc​Js(ρ')ejkt,pq⋅ρ' dS'.

Rather than hard-coding a free-space far-field expression, a robust layered-medium implementation should obtain each upward- and downward-propagating modal amplitude from the spectral dyadic Green function,

Epqscat,±=Kpq ±⋅Jpq,

where the operator Kpq ± contains the TE/TM modal response of the multilayer environment. This construction naturally allows the substrate and different upper/lower media to be handled consistently with the layered Green-function formulation [4]. [12]

For a layered background, the total outgoing mode generally contains both the response of the unpatterned background and the field radiated by the induced current. Thus,

Epqref=Epqbg,ref+Epqscat,-,

Epqtr=Epqbg,tr+Epqscat,+.

Neglecting the background term is a common source of incorrect transmission coefficients when a conducting pattern is placed on a dielectric slab.

Using power-normalized Floquet waves, the fundamental-mode scattering parameters are

S11=b1,00a1,00,  S21=b2,00a1,00.

If raw transverse electric-field amplitudes rather than power waves are used, impedance/admittance normalization is required. For example,

S21=ReY2,00ReY1,00 E00trE00inc,

where the Floquet modal admittances for an isotropic medium are

YpqTE=kz,pqωμ,  YpqTM=ωϵkz,pq.

Equation (20) is especially important when the media on the two sides of the metasurface differ: the ratio of electric-field amplitudes is then not, by itself, a power-normalized S21.

When the cell period is sufficiently subwavelength that only the (0,0) order propagates, S11 and S21 provide the conventional specular reflection and transmission channels. Once a higher Floquet order becomes propagating, however, a two-number description is incomplete. The implementation should then calculate a multimode set Spq and use

R=(p,q)∈P1​PpqrefPinc,  T=(p,q)∈P2​PpqtrPinc,

where P1 and P2 contain the propagating reflected and transmitted modes. This distinction should be tested near the first diffraction/Rayleigh threshold rather than assuming that the fundamental mode is always the only propagating one.

For a passive lossless test structure, a useful implementation check is

R+T≃1,

whereas for lossy materials

R+T+A≃1.

These conservation tests are extremely valuable because an apparently plausible S11 curve can still conceal incorrect modal normalization, Green-function truncation, or transmission-reference conventions.

## GPU-accelerated solver architecture

Draft manuscript text—GPU Implementation.
The numerical workflow is separated into geometry preprocessing, periodic interaction evaluation, impedance-matrix assembly, linear solution, and modal post-processing. The principal GPU opportunity arises from the large number of independent source-testing interactions required to form Z. Earlier GPU-MoM research established that matrix construction and linear algebra are viable GPU targets, including RWG formulations and CUDA implementations [7]–[11]. [15]

For N unknowns, the dense matrix contains N2 complex entries. A GPU kernel can therefore map testing/source pairs (m,n) to independent threads or thread blocks, with quadrature data, basis-function geometry, periodic phase terms, and Green-function parameters stored in device-accessible arrays. The self and near-singular interactions should be separated from ordinary far interactions when different quadrature or singularity-extraction procedures are required. This separation is numerically more defensible than forcing all matrix entries through a single vectorized kernel merely to maximize GPU occupancy.

A practical architecture is

geometry→mesh/basis→periodic-GF data→Z→V→I→Floquet projection→{Spq}.

The CPU is well suited to lightweight geometry bookkeeping and job orchestration, while the expensive interaction loops and dense linear algebra remain resident on the accelerator. Host-device transfers should be minimized; in particular, transferring Z repeatedly between the host and device would negate part of the benefit of GPU matrix assembly.

For a dense direct solver,

ZI=V

may be solved through LU factorization with pivoting. GPU LU decomposition has previously been investigated explicitly for large MoM systems [9], while modern GPU linear-algebra libraries provide dense factorizations and linear-system solves directly on the accelerator. NVIDIA's cuSOLVER documentation describes GPU-accelerated dense and sparse matrix decompositions and solution routines, including LU-related functionality. [16]

Two implementation paths are particularly appropriate for a Python codebase. CuPy provides a NumPy/SciPy-like GPU array model and is a natural choice when the existing CPU implementation already uses NumPy semantics. PyTorch is attractive when longer-term integration with machine-learning code or differentiability is desired; current PyTorch supports complex tensors and automatic differentiation for complex-valued operations. These are implementation options rather than research contributions in themselves. [17]

A useful software design is therefore to separate the electromagnetic formulation from the numerical backend:

backend∈{NumPy,CuPy,PyTorch}.

The CPU and GPU implementations can then share the same basis-function definitions, quadrature nodes, periodic Green-function conventions, and S-parameter post-processing. This is valuable scientifically because differences between CPU and GPU results can be attributed primarily to numerical precision and execution backend rather than to two independent physics implementations.

The paper should report both complex64 and complex128 behavior if single precision is contemplated. Resonant metasurface problems may produce poorly conditioned matrices or highly sensitive phase responses, and a large speed advantage obtained only by reducing precision is not meaningful unless the resulting S-parameter error is quantified. The production default should therefore be justified empirically rather than chosen from GPU throughput specifications.

### Batch processing and HPC deployment

The most important high-throughput parallelism may occur above the individual MoM matrix. A dataset-generation task containing independent unit cells

{G1,G2,…,GM}

naturally decomposes into independent forward solves. Consequently, one GPU can process a queue or batch of geometries while multiple GPUs process different queues simultaneously. This form of coarse-grained parallelism avoids inter-GPU communication and is likely to be more efficient for moderate unit-cell systems than distributing every single dense matrix across multiple devices. This is an architectural inference to be tested by the proposed benchmarks, not a claim that one strategy is universally optimal.

Several levels of parallelism are therefore available:

GPU threads⏟matrix interactions ⊂ GPU process⏟one/batched MoM solves ⊂ node⏟several GPUs ⊂ cluster⏟geometry/frequency jobs.

For HPC deployment, a simple and robust initial model is one worker process per GPU combined with SLURM/PBS job arrays over geometry batches. A distributed dense solver becomes more relevant only when a single MoM system no longer fits into one device's memory. Current NVIDIA tooling includes cuSOLVERMp and Python-accessible distributed direct linear algebra for dense systems across multiple GPUs/nodes, demonstrating that this extension is technically possible if required. [18]

Frequency sweeps require additional care. The Green function and hence Z generally change with frequency, so a single factorization cannot ordinarily be reused over an entire frequency sweep. However, for multiple excitations at identical frequency, geometry, and Floquet phase—such as multiple right-hand sides compatible with the same operator—a single matrix factorization can be reused. Conversely, changing incidence angle changes the Floquet phase and normally changes the periodic operator, so an angle sweep should not be treated as a simple new right-hand side unless the particular formulation demonstrates otherwise.

Variable geometry also complicates conventional “tensor batching.” If every geometry has identical topology and number of unknowns, several matrices can be formed as a batch and passed to batched GPU routines. If geometries have different meshes and different N, job-level batching is usually cleaner than padding all matrices to the largest dimension. The paper should distinguish these two meanings of “batch processing.”

## S-parameter validation and expected results

Accuracy should be demonstrated independently of performance. A fast solver whose S-parameters have only been visually compared with one commercial-software plot is not sufficient evidence for an electromagnetic solver paper. Validation should proceed from numerical self-consistency to canonical periodic cases and finally to a parametrically diverse comparison against an established full-wave solver.

The first benchmark should be a simple free-standing periodic conducting patch or aperture array of the class historically treated with periodic MoM [2]. The second should include a dielectric substrate, thereby exercising the layered Green function [4]. Further cases should vary unit-cell period, feature dimensions, dielectric permittivity and thickness, polarization, and incidence angle. If the implementation supports resistive/impedance surfaces, at least one lossy example should be included. [20]

Commercial-software comparisons must use identical physical assumptions: the same lattice dimensions, substrate thickness, relative permittivity and loss tangent, conductor approximation, incidence angle, polarization, periodic boundary conditions, Floquet-port definitions, port reference planes, and de-embedding conventions. An apparent phase discrepancy can otherwise be caused by different reference planes rather than by an error in the MoM formulation.

For frequency samples fi, a complex error metric can be defined as

RMSES=12Nfi=1NfS11MoM(fi)-S11ref(fi)2+S21MoM(fi)-S21ref(fi)2.

A normalized error can additionally be reported as

ϵS=∥SMoM-Sref∥2∥Sref∥2.

Magnitude-only agreement is inadequate because many metasurface applications exploit transmission or reflection phase. Accordingly, the plots should include both magnitude and unwrapped phase. Phase errors close to transmission or reflection nulls should, however, be interpreted cautiously because the phase becomes poorly conditioned as the complex amplitude approaches zero.

For resonant structures, report the relative shift in resonance frequency,

ϵfr=|frMoM-frref|frref,

together with resonance depth/peak error and bandwidth error where applicable.

### Numerical convergence

Mesh convergence should be demonstrated using at least three discretizations,

N1<N2<N3,

with

ΔS(k)=maxfS(Nk)(f)-S(N3)(f).

The reference commercial simulation must also be independently converged; otherwise agreement between an unconverged CST/HFSS result and an unconverged MoM result has little meaning.

The periodic Green-function computation requires a second convergence study independent of mesh size. If a spectral truncation, lattice cutoff, Ewald parameter, or hybrid spatial/spectral cutoff is used, S11 and S21 should be plotted against that parameter until the desired tolerance is reached. The need to accelerate or carefully control periodic Green-function sums is established in the periodic-integral-equation literature [3], and recent metasurface MoM work continues to focus on this computational component [6]. [21]

Energy balance provides another solver-level check:

δP=1-(R+T+A).

For a reciprocal two-port configuration in which the relevant port and symmetry conditions apply, reciprocity can additionally be tested through the appropriate equality of forward and reverse scattering coefficients. These checks should complement, not replace, the cross-solver comparison.

### Performance benchmarking

Performance should be separated into

ttotal=tmesh+tGF+tfill+tsolve+tpost.

Reporting only a highly parallel kernel's speed-up can materially overstate the benefit experienced by a user. Earlier GPU-MoM publications reported acceleration of matrix filling, matrix-vector operations, and linear algebra as distinct stages, which makes stage-level profiling a well-established comparison strategy [7]–[11]. [22]

Define end-to-end acceleration as

ΓGPU=tCPU,totaltGPU,total.

For dataset generation, however, an even more useful metric is throughput,

T=Nunit cellstwall,

reported as unit cells per hour or frequency points per second. This metric directly connects the CEM contribution to the machine-learning motivation.

The CPU/GPU comparison must specify processor, number of CPU threads, GPU model, available memory, complex precision, software versions, matrix dimensions, number of frequencies, and whether GPU initialization/JIT compilation is included. Warm-start and cold-start timing should be distinguished when compilation or kernel caching is present.

### Expected Results and Discussion

A useful IEEE results table is:

| Test case | Unknowns N | Frequency samples | Complex S-RMSE | Resonance shift | CPU time | GPU time | Speed-up |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Free-standing patch | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Printed patch | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Geometry sweep case | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Oblique-incidence case | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |

The corresponding manuscript discussion can be drafted now without inventing data:

Accuracy. Figure [X] compares the complex reflection and transmission coefficients obtained using the proposed periodic MoM solver with those obtained from [CST Studio Suite/HFSS]. The two solutions exhibit consistent resonance locations and spectral trends over [frequency range]. The resulting complex S-parameter RMSE is [value], while the relative displacement of the dominant resonance is [value]. The largest deviations occur near [frequency/physical regime], where [mesh resolution/Green-function convergence/material approximation] has the strongest influence.

Convergence. Figure [X] shows the variation of S11 and S21 with increasing mesh density. Beyond [number] unknowns, the response changes by less than [value] according to the convergence metric in (32). An independent convergence study of the periodic Green-function truncation is shown in Fig. [X], establishing the numerical parameters used in subsequent simulations.

GPU acceleration. For the largest test problem containing [number] unknowns, impedance-matrix construction requires [CPU time] on the CPU and [GPU time] on the GPU. The corresponding linear-solution times are [values], producing an end-to-end acceleration of [speed-up]. Matrix assembly accounts for [percentage] of the CPU runtime and [percentage] of the GPU runtime.

Dataset throughput. When [number] independent unit-cell simulations are processed in batches on [GPU configuration], the implementation sustains [unit cells/hour] or [frequency points/s]. Scaling from one to [number] GPUs changes aggregate throughput from [value] to [value], corresponding to a parallel efficiency of [value].

Memory scaling. The maximum system evaluated contains [number] unknowns and requires [GPU memory] of device memory. The measured memory scaling is consistent with the dense-matrix storage expected from (28). Problems exceeding [number] unknowns therefore require [chunking/iterative solution/multi-GPU decomposition], which constitutes a current limitation of the implementation.

Those statements become publishable only after filling them with measured values.

## AI-enabled inverse design and conclusion

Draft manuscript text—Application to Data-Driven Metasurface Design.
A central motivation for the proposed solver is that electromagnetic simulation increasingly acts as a data-generation layer beneath machine-learning models. Deep neural networks have been trained both as fast forward approximators and as inverse-design models in nanophotonics [14], [16]. Generative networks have also been used to map desired spectral responses into candidate metasurface geometries [15]. These approaches can reduce the cost of inference or optimization after training but do not eliminate the need for trustworthy electromagnetic labels during model construction. [9]

The proposed MoM engine can participate in such workflows at three different levels. The first is offline dataset generation, where geometries are sampled from a parameterized design space and simulated in parallel to produce

D={(xi,yi)}i=1M,  yi=S11(f1),…,S11(fK),S21(f1),…,S21(fK).

Here xi describes geometry and material parameters. Storing the real and imaginary parts of S, rather than only |S|, preserves both amplitude and phase information for subsequent learning.

The second mode is online physics validation. An inverse network or generative model proposes candidate geometry xg; the MoM solver evaluates

SMoM=FEM(xg),

and a physical objective,

Lphys=dSMoM,Starget,

is used to rank, reject, or refine the candidate. This does not require the MoM solver to be differentiable. Consequently, the present implementation can legitimately be described as compatible with physics-guided inverse-design loops even if it is currently called as a black-box forward simulator.

A useful extension is active learning: the surrogate proposes or identifies informative geometries, the GPU MoM engine supplies high-fidelity labels for those designs, and the model is retrained. This strategy concentrates electromagnetic computation where the learned model is uncertain instead of uniformly sampling the entire geometry space.

The third and more demanding mode is a differentiable electromagnetic layer. TORCWA already demonstrates the practical value of combining GPU electromagnetic simulation with automatic differentiation for metasurface optimization, although it uses RCWA rather than MoM [13]. PyTorch itself supports complex-valued autograd, so a PyTorch implementation provides relevant software infrastructure, but that alone does not make a MoM code differentiable with respect to geometry. [23]

For the MoM system

Z(p)I(p)=V(p),

differentiation with respect to a continuous design parameter p gives

dIdp=Z-1dVdp-dZdpI.

Equation (41) shows why true differentiability requires derivatives of the geometry-dependent impedance matrix and excitation, not merely an autograd-enabled matrix solve. Fixed-topology parametric meshes are therefore much easier to differentiate than designs in which the connectivity itself changes. An adjoint formulation could eventually reduce the number of solves required when the number of geometry parameters is large.

This distinction should be reflected in the conference paper's wording:

Safe present-tense claim:
“The Python interface allows the solver to be incorporated as a physics evaluator within machine-learning and generative inverse-design workflows.”

Claim to make only after implementation and gradient verification:
“The proposed solver provides differentiable electromagnetic simulation with respect to metasurface geometry.”

The motivation is strengthened by published examples in which full-wave simulation is explicitly part of dataset generation. Ghorbani et al. generated metasurface responses using CST as part of a neural-network design workflow [17]. Thus, a specialized internal solver can remove a commercial full-wave call from every training sample while preserving the option to use commercial software to validate a statistically representative subset. [10]

The comparison with TORCWA is worth discussing directly rather than avoiding it. RCWA/Fourier-modal methods are highly natural for periodic structures, and TORCWA already demonstrates GPU execution and gradient-based optimization [13]. The proposed MoM approach is differentiated not by the general idea of GPU metasurface simulation but by the integral-equation representation: for thin conducting patterns, the unknowns can be placed directly on the current-carrying surface. The paper should present this as a complementary numerical route, not as a universal superiority claim. [24]

### Conclusion and Future Work

Draft conclusion.
This work presents a Python-native, GPU-compatible periodic Method of Moments framework for high-throughput electromagnetic characterization of metasurface unit cells. The solver combines [RWG/actual basis] current discretization with a Floquet-periodic [free-space/layered-medium] Green-function formulation and GPU-accelerated impedance-matrix assembly and linear solution. Reflection and transmission coefficients are recovered through projection of the solved currents onto propagating Floquet modes, permitting direct prediction of complex S11 and S21.

Validation against [CST Studio Suite/HFSS] over [number] structures and [frequency range] gives [accuracy metrics]. Relative to the CPU implementation, the GPU formulation reduces total simulation time from [value] to [value], corresponding to an end-to-end acceleration of [value]. On [number/type] GPUs, the resulting implementation achieves a dataset-generation throughput of [value] unit cells per hour.

The principal value of the implementation lies in combining a specialized periodic integral-equation solver with a high-throughput Python execution model. Once validated, the same solver can generate electromagnetic labels for surrogate or generative models and can evaluate candidate structures directly inside inverse-design loops without requiring a commercial full-wave simulation for every iteration. Commercial solvers remain important for independent validation and for structures outside the assumptions of the present surface-integral formulation.

Future work will address [dielectric-volume/PMCHWT support], accelerated periodic Green-function evaluation, iterative or compressed matrix methods for larger systems, multi-GPU execution, adaptive data generation, and—subject to validation of geometry derivatives—a differentiable or adjoint MoM formulation for gradient-based inverse design.

## Research gap, experiments, and claims requiring validation

### Research-gap table

| Approach | What is already established | GPU/HPC status | Python/ML integration | Unit-cell S-parameter focus | Gap relative to proposed work |
| --- | --- | --- | --- | --- | --- |
| Commercial CST/HFSS-class solver | General full-wave modeling, periodic boundaries, Floquet ports, broad material/geometry support | Commercial products support high-performance execution, subject to vendor infrastructure/licensing | Automation APIs exist, but workflow remains tied to proprietary software | Strong | Excellent reference solver, but not a self-controlled CEM engine for unrestricted internal data-generation workflows |
| Conventional periodic MoM [2]–[5] | Periodic integral equations, periodic Green functions, layered media, FSS/array analysis | Historically CPU-oriented formulations | Not generally designed around modern ML pipelines | Yes or closely related | Does not by itself provide GPU/HPC/ML-oriented production workflow |
| GPU MoM [7]–[11] | GPU matrix filling, RWG interactions, linear algebra and scattering calculations already demonstrated | Strong single-GPU precedent | Mostly C/CUDA-era CEM implementations | Not generally the principal emphasis of landmark GPU papers | GPU acceleration alone is not novel |
| Recent periodic metasurface MoM [6] | Efficient periodic-MoM treatment remains an active research topic | Computational acceleration is an explicit objective | Not presented as a Python ML-dataset platform | Periodic metasurface analysis | Prevents claiming novelty simply for “efficient periodic MoM for metasurfaces” |
| Bempp-cl [12] | Python boundary-element framework; Maxwell applications | OpenCL CPU/GPU execution | Strong Python accessibility | Not a purpose-built periodic metasurface unit-cell S-parameter pipeline | Prevents claiming “first Python GPU integral-equation EM solver” |
| TORCWA [13] | Periodic metasurface simulation, GPU acceleration, gradients and optimization | GPU-native | Strong PyTorch/inverse-design integration | Yes, via RCWA | Closest conceptual competitor, but numerical formulation is Fourier-modal rather than MoM |
| Proposed framework | Periodic surface MoM specialized for the target structure class | GPU matrix assembly/solve plus coarse-grained multi-GPU jobs | Python-native data generation and physics-loop API | Direct design objective | Contribution is the integrated specialization and demonstrated throughput/accuracy |

The underlying literature supporting this comparison spans established periodic MoM [2]–[5], the recent periodic metasurface solver of Dessy et al. [6], GPU-MoM work [7]–[11], Bempp-cl [12], and TORCWA [13]. [25]

### Experiments that should be completed before submission

| Research question | Required experiment | What to report |
| --- | --- | --- |
| Is the EM formulation correct? | Free-standing periodic PEC patch/aperture benchmark | Complex S11, S21, convergence and energy balance |
| Does the layered Green function work? | Patch/FSS on one or more dielectric layers | Magnitude and phase versus CST/HFSS |
| Is the mesh converged? | At least three mesh densities | S-parameter convergence versus N |
| Is the periodic sum converged? | Vary spectral/lattice/Ewald/hybrid cutoff | Error versus Green-function tolerance |
| Does the solver work only at normal incidence? | Several (θ,ϕ) values | TE and TM S-parameters |
| Are higher diffraction orders handled correctly? | Sweep through a Floquet cutoff/Rayleigh anomaly | Modal Spq and total R+T+A |
| Is polarization handled correctly? | TE, TM, and—if supported—cross-polarized/an-isotropic structures | Co- and cross-polar S-parameters |
| Is the CPU and GPU physics identical? | Execute the same mesh/quadrature in both backends | Maximum CPU/GPU complex difference |
| Does reduced precision remain accurate? | complex64 versus complex128 | Error, time, and memory |
| Where does GPU acceleration arise? | Profile assembly, solve and post-processing separately | Stage timing plus end-to-end timing |
| Does performance scale with problem size? | Sweep N over as wide a range as GPU memory allows | Time and memory versus N |
| Does batching help? | Sweep number of simultaneous/queued cells | Throughput versus batch size |
| Does HPC deployment actually scale? | One, two, four, … GPUs or available cluster sizes | Unit cells/hour, speed-up, parallel efficiency |
| Is it useful for ML rather than merely fast CEM? | Generate a substantial training set and train one surrogate | Held-out prediction error |
| Is solver-generated ML data trustworthy? | Train on MoM labels; test on unseen CST/HFSS designs | ML error relative to commercial full-wave reference |
| Can it guide inverse design? | Generate candidates with a surrogate/GAN/diffusion model and rescore with MoM | Target-vs-realized S response and refinement success |

The last two experiments would substantially strengthen the novelty claim. Without them, the paper is fundamentally a GPU computational-electromagnetics paper whose AI motivation is prospective. With even one convincing surrogate-training or inverse-design demonstration, the connection between the solver architecture and the stated machine-learning motivation becomes experimentally demonstrated rather than rhetorical.

A particularly strong conference experiment would be:

Generate [104–105] candidate unit cells with GPU-MoM→train a surrogate→evaluate held-out candidates→validate a random subset with CST

where the actual dataset size should be chosen from your computational budget rather than inserted merely to make the paper appear large.

### Claims that need especially careful wording

| Proposed claim | Risk | Recommended wording/evidence |
| --- | --- | --- |
| “The first GPU MoM solver” | Incorrect; GPU MoM predates this work [7]–[11] | Do not use |
| “First GPU periodic MoM” | High-risk novelty claim | Avoid unless an exhaustive database search establishes it |
| “First Python GPU EM solver” | Incorrect in broad form because software such as Bempp-cl exists [12] | Say “Python-native implementation specialized for…” |
| “First GPU metasurface solver” | Incorrect in broad form; TORCWA exists [13] | Explicitly distinguish MoM from RCWA |
| “First efficient MoM solver for periodic metasurfaces” | Very high risk after Dessy et al. 2025 [6] | Do not use |
| “Accurate” | Unsupported until convergence/cross-validation exists | Give quantitative complex S-errors |
| “Faster” | Ambiguous | Report same-hardware CPU/GPU end-to-end timing |
| “Massively faster” | Marketing language | Replace with measured speed-up |
| “HPC scalable” | Unsupported by running on one GPU | Show throughput with GPU count or say “HPC-deployable” |
| “Open-source” | False unless code is actually released under an explicit license | Use “Python-based” until release |
| “Eliminates CST/HFSS” | Overclaim | State that it reduces reliance on commercial solvers for repetitive in-scope simulations while using them for validation |
| “Commercial GPU licenses are more expensive” | Contract-dependent | State that commercial cluster use is subject to proprietary licensing; quantify only using your institution's actual terms |
| “Physics-guided inverse design” | Potentially vague | Demonstrate a candidate→MoM→objective/refinement loop |
| “Differentiable MoM” | Requires geometry gradients, not merely PyTorch | Claim only after finite-difference/adjoint gradient verification |
| “General metasurface solver” | Too broad if solver assumes thin PEC sheets | Explicitly define supported conductors, dielectrics, substrates, incidence and diffraction regime |
| “Large-scale data generation” | Undefined | Report unit cells/hour, total generated samples, GPU-hours and memory |

The literature makes the first five cautions especially important. GPU MoM [7]–[11], periodic metasurface MoM [6], Python accelerated BEM [12], and GPU/PyTorch metasurface optimization [13] are all established precedents. [26]

A strong final novelty sentence, after your experiments have been completed, would be:

“The novelty of the proposed solver lies not in GPU acceleration of MoM alone, but in the development and validation of a Python-native periodic-MoM workflow in which GPU-parallel electromagnetic assembly, Floquet-mode S-parameter extraction, batch-oriented HPC execution, and direct machine-learning interfaces are co-designed for high-throughput metasurface unit-cell simulation.”

That statement is substantially more defensible than a “first GPU MoM” claim.

## References

[1] S. M. Rao, D. R. Wilton, and A. W. Glisson, “Electromagnetic scattering by surfaces of arbitrary shape,” IEEE Transactions on Antennas and Propagation, vol. 30, no. 3, pp. 409–418, May 1982, doi: 10.1109/TAP.1982.1142818. This is the foundational RWG surface-basis paper. [14]

[2] T. Cwik and R. Mittra, “Scattering from a periodic array of free-standing arbitrarily shaped perfectly conducting or resistive patches,” IEEE Transactions on Antennas and Propagation, vol. 35, no. 11, pp. 1226–1234, Nov. 1987, doi: 10.1109/TAP.1987.1143999. [27]

[3] S. Singh, W. F. Richards, J. R. Zinecker, and D. R. Wilton, “Accelerating the convergence of series representing the free space periodic Green's function,” IEEE Transactions on Antennas and Propagation, vol. 38, no. 12, pp. 1958–1962, Dec. 1990, doi: 10.1109/8.60985. [28]

[4] K. A. Michalski and J. R. Mosig, “Multilayered media Green's functions in integral equation formulations,” IEEE Transactions on Antennas and Propagation, vol. 45, no. 3, pp. 508–519, Mar. 1997, doi: 10.1109/8.558666. [12]

[5] X. Dardenne and C. Craeye, “Method of Moments simulation of infinitely periodic structures combining metal with connected dielectric objects,” IEEE Transactions on Antennas and Propagation, vol. 56, no. 8, pp. 2372–2380, Aug. 2008, doi: 10.1109/TAP.2008.926779. [29]

[6] J. Dessy, L. Paucot, D. Tihon, M. Bodehou, and C. Craeye, “Efficient Method of Moments analysis of periodic metasurfaces combining spectral and spatial domain approaches,” in Proc. URSI International Symposium on Electromagnetic Theory (EMTS), Bologna, Italy, Jun. 2025, pp. 1–4, doi: 10.46620/URSIEMTS25/BHHQ9781. This recent paper is particularly important for novelty positioning because it directly treats efficient periodic MoM for metasurfaces. [30]

[7] S. Peng and Z. Nie, “Acceleration of the Method of Moments calculations by using graphics processing units,” IEEE Transactions on Antennas and Propagation, vol. 56, no. 7, pp. 2130–2133, Jul. 2008. [31]

[8] E. Lezar and D. B. Davidson, “GPU-accelerated Method of Moments by example: Monostatic scattering,” IEEE Antennas and Propagation Magazine, vol. 52, no. 6, pp. 120–135, Dec. 2010, doi: 10.1109/MAP.2010.5723240. [32]

[9] E. Lezar and D. B. Davidson, “GPU-based LU decomposition for large Method of Moments problems,” Electronics Letters, vol. 46, no. 17, pp. 1194–1196, 2010, doi: 10.1049/el.2010.1680. [33]

[10] T. Topa, A. Noga, and A. Karwowski, “Adapting MoM with RWG basis functions to GPU technology using CUDA,” IEEE Antennas and Wireless Propagation Letters, vol. 10, pp. 480–483, 2011, doi: 10.1109/LAWP.2011.2154373. [34]

[11] Y. Shi and J. M. Jin, “GPU-accelerated time-domain marching-on-in-degree solution,” Electronics Letters, vol. 49, no. 6, pp. 393–394, Mar. 2013, doi: 10.1049/el.2012.4227. The work is time-domain rather than the proposed frequency-domain formulation, but it is useful evidence for GPU acceleration of integral-equation matrix filling and matrix-vector operations. [35]

[12] T. Betcke and M. W. Scroggs, “Bempp-cl: A fast Python based just-in-time compiling boundary element library,” Journal of Open Source Software, vol. 6, no. 59, Art. no. 2879, 2021, doi: 10.21105/joss.02879. Bempp documentation also includes Maxwell problems and OpenCL execution on CPU/GPU devices, making this an important prior art reference for Python-based accelerated integral-equation software. [36]

[13] C. Kim and B. Lee, “TORCWA: GPU-accelerated Fourier modal method and gradient-based optimization for metasurface design,” Computer Physics Communications, vol. 282, Art. no. 108552, Jan. 2023, doi: 10.1016/j.cpc.2022.108552. This is a key neighboring reference because it combines Python/PyTorch, GPU electromagnetic simulation, and gradient-based metasurface optimization, although using RCWA rather than MoM. [37]

[14] D. Liu, Y. Tan, E. Khoram, and Z. Yu, “Training deep neural networks for the inverse design of nanophotonic structures,” ACS Photonics, vol. 5, no. 4, pp. 1365–1369, 2018, doi: 10.1021/acsphotonics.7b01377. [38]

[15] Z. Liu, D. Zhu, S. P. Rodrigues, K.-T. Lee, and W. Cai, “Generative model for the inverse design of metasurfaces,” Nano Letters, vol. 18, no. 10, pp. 6570–6576, 2018, doi: 10.1021/acs.nanolett.8b03171. [39]

[16] J. Peurifoy, Y. Shen, L. Jing, Y. Yang, F. Cano-Renteria, B. G. DeLacy, J. D. Joannopoulos, M. Tegmark, and M. Soljačić, “Nanophotonic particle simulation and inverse design using artificial neural networks,” Science Advances, vol. 4, no. 6, Art. no. eaar4206, 2018, doi: 10.1126/sciadv.aar4206. [40]

[17] F. Ghorbani, S. Beyraghi, J. Shabanpour, H. Oraizi, H. Soleimani, and M. Soleimani, “Deep neural network-based automatic metasurface design with a wide frequency range,” Scientific Reports, vol. 11, Art. no. 7102, 2021, doi: 10.1038/s41598-021-86588-2. This paper is especially relevant to the proposed motivation because its data-driven metasurface workflow uses full-wave simulation to generate electromagnetic training responses. [10]

[1] [15] [22] [31] Acceleration of the Method of Moments Calculations by Using ...

https://ieeexplore.ieee.org/document/4558314/

[2] [7] [20] [25] [27] Scattering from a periodic array of free-standing arbitrarily ...

https://ieeexplore.ieee.org/document/1143999/

[3] [36] Bempp-cl: A fast Python based just-in-time compiling boundary ...

https://joss.theoj.org/papers/10.21105/joss.02879

[4] [11] [26] [30] Efficient Method of Moments Analysis of Periodic ...

https://ieeexplore.ieee.org/abstract/document/11097789/

[5] The SIMULIA Unified Licensing Model - Dassault Systèmes

https://discover.3ds.com/simulia-unified-licensing

[6] [29] Method of Moments Simulation of Infinitely Periodic Structures ...

https://ieeexplore.ieee.org/document/4589094/

[8] [14] Electromagnetic scattering by surfaces of arbitrary shape - IEEE Xplore

https://ieeexplore.ieee.org/document/1142818/

[9] [38] Training Deep Neural Networks for the Inverse Design of ...

https://pubs.acs.org/apchd5/article/5/4/1365/684462/Training-Deep-Neural-Networks-for-the-Inverse

[10] Deep neural network-based automatic metasurface design with ...

https://www.nature.com/articles/s41598-021-86588-2

[12] Multilayered media Green's functions in integral equation ...

https://ieeexplore.ieee.org/document/558666/

[13] [21] [28] Accelerating the convergence of series representing ...

https://ieeexplore.ieee.org/document/60985/

[16] [19] [33] Efficient out‐of‐GPU memory strategies for solving matrix equation ...

https://ietresearch.onlinelibrary.wiley.com/doi/abs/10.1049/el.2015.2175

[17] Overview — CuPy 14.2.0 documentation

https://docs.cupy.dev/en/stable/overview.html

[18] cuSOLVERMp

https://docs.nvidia.com/cuda/cusolvermp/

[23] [24] [37] TORCWA: GPU-accelerated Fourier modal method and ...

https://www.sciencedirect.com/science/article/abs/pii/S0010465522002715

[32] GPU-Accelerated Method of Moments by Example - IEEE Xplore

https://ieeexplore.ieee.org/document/5723240/

[34] Adapting MoM With RWG Basis Functions to GPU Technology ...

https://ieeexplore.ieee.org/document/5766704/

[35] GPU‐accelerated time‐domain marching‐on‐in‐degree solution

https://ietresearch.onlinelibrary.wiley.com/doi/10.1049/el.2012.4227

[39] Generative Model for the Inverse Design of Metasurfaces

https://pubs.acs.org/nalefd/article/18/10/6570/1240495/Generative-Model-for-the-Inverse-Design-of

[40] Nanophotonic particle simulation and inverse design using artificial ...

https://www.science.org/doi/10.1126/sciadv.aar4206